SWARNANDHRA COLLEGE OF ENGINEERING & TECHNOLOGY

(AUTONOMOUS)

DEPARTMENT OF CIVIL ENGINEERING

QUESTION BANK

SUBJECT: STRUCTIRAL ANALYSIS-I

Unit-I                       Propped cantilever

- Write the difference between statically determinate and indeterminate structure?

- What do you understand by the term prop?

- Differentiate between cantilever and propped cantilever?

- What is the degree of indeterminacy of a propped cantilever?

- What are the steps involved in the analysis of propped cantilever?

- Derive an expression for the prop reaction in a cantilever carrying a u.d.l over the entire span and propped at the free end.

- A cantilever of length ‘L’ carries a concentrated load ‘W’ at its mid-span. If the free end is supported by a prop, find the reaction at the prop and also draw the S.F. and B.M. diagrams.

- A cantilever of length 4m carries a uniformly distributed load of 1kN/m length over the whole length .The free end of the cantilever is supported on a prop. If E = 2 x 105 N/ mm2 and I = 108 mm 4 , then (i) find the prop reaction (ii) deflection at the centre of cantilever.

- A cantilever of 6m length carries an U.D.L of 12 kN/m over the full span. If the free end is supported by a prop, find the reaction at the prop and also draw the S.F. and B.M. diagrams.

- Analyse the propped cantilever beam loaded as shown in the Figure. Draw the S.F.D and B.M.D. Assume EI constant throughout.

- A cantilever of length 6m carries a u.d.l of 2 KN/m over a length of 4m starting from the fixed end. The cantilever is propped rigidly at the free end. If the value of E=2x105 N/mm2 and I=108 mm4 then determine: a) Reaction at the rigid drop, b) The deflection at the centre of the cantilever and c) Magnitude and position of maximum deflection.

- A propped cantilever beam is shown in figure. Calculate the prop Reaction and also draw the BM & SF diagrams.

SWARNANDHRA COLLEGE OF ENGINEERING & TECHNOLOGY

(AUTONOMOUS)

DEPARTMENT OF CIVIL ENGINEERING

QUESTION BANK

SUBJECT: STRUCTIRAL ANALYSIS-I

Unit- 2         		    Fixed beams

- What are the advantages of fixed beams?

- How fixed beams can be statically determinate?

- Draw the S.F.D for a fixed beam carrying eccentric load?

- A fixed beam AB of 3m span is subjected to a point load of 15kN at a distance of span. Find the fixing moments and deflection of the beam under the load. Take EI = 2 x 103 kN-m2.

- A fixed beam of span 6 m is subjected a UDL of 5 kN/m on the left half of the span and a point load of 15 kN at the middle of the right half of the span. Draw the S.F. and B.M. diagrams.

- A fixed beam of span 8 m is subjected to a linearly varying load of 8 kN/m from one support to 6 kN/m to the other support. Find the support reactions and moments. Draw the shear force and bending moment diagrams.

- A fixed beam of 6 m span carries a uniformly distributed load of 12 kN/m run over the whole span. The level of right hand support sinks by 8 mm below that the left hand end. Take E=2.10x108kN/m2 and I=4.50x10-5 m4. Find (i) Support moments, (ii) Support reactions, and (iii) Deflection at the centre.

- A fixed beam is shown in figure, analyze the beam and draw the SF and BM diagram

SWARNANDHRA COLLEGE OF ENGINEERING & TECHNOLOGY

(AUTONOMOUS)

DEPARTMENT OF CIVIL ENGINEERING

QUESTION BANK

SUBJECT: STRUCTIRAL ANALYSIS-I

Unit-3 			Continuous beams

- Define a continuous beam?

- State Claypeyron’s Three Moment theorem?

- Write the equations of the Claypeyron’s theorem with sinking of supports?

- A continuous beam ABC is simply supported at A and C and continuous over support B with AB = 5m and BC = 6m.Auniformly distributed load of 12kN/m is acting over the beam. The moment of inertia is I throughout the span. Analyse the continuous beam and draw S.F.D and B.M.D.

- Analyze the continuous beam shown in figure, using three-moment equation. Draw S.F and B.M diagrams.

- A continuous beam ABC consists of two spans AB of length 4m, and BC of length 3m. The span AB carries a point load of 100 KN at its middle points. The span BC carries a point load of 120 KN at 1m from C. The end A is fixed and the end C is simply supported. Find The moments at the supports The reactions at the supports and Draw the B.M diagram. Use Clapeyron’s theorem of three moments.

- A continuous beam ABCD 18 m long is loaded as shown in figure. During loading support B sinks by 10 mm. Find support moments and plot shear force and bending moment diagrams for the beam. Take E = 20 kN/mm2 , I = 8x106 mm4.

- Draw the Shear force and bending moment diagram for the beam shown in figure. Use Clapeyron’s theorem of three moments. EI=1x105 N/mm2.

- A continuous beam ABCD is simply supported over three spans. Span AB is 9m carrying an udl of 5kN/m, span BC is 14M carrying an udl of 5kN/m and span CD is 6M carrying an udl of 8kN/m. Find the moment over supports B and C.Draw B.M.D.

- Derive the Clayperon’s theorem of three moments.

- A continuous beam ABC is simply supported at A and C and continuous over support B with AB = 5m and BC = 6m.Auniformly distributed load of 12kN/m is acting over the beam. The moment of inertia is I throughout the span. Analyse the continuous beam and draw S.F.D and B.M.D.

- Abeam ABCD 9.2m long is fixed at A and is supported at B and C at distances 4m and 7m from A with an overhang CD 2.2m long. The span AB carries a point load of 32kN at the mid span. A point load of 16kN acts at the end D. Find the moments and reactions at the supports.

SWARNANDHRA COLLEGE OF ENGINEERING & TECHNOLOGY

(AUTONOMOUS)

DEPARTMENT OF CIVIL ENGINEERING

QUESTION BANK

SUBJECT: STRUCTIRAL ANALYSIS-I

Unit-4 									Slope- Deflection method

- Define slope deflection method?

- State the assumptions made in the slope deflection methods?

- Write the steps involved in slope deflection method?

- What are the sign conventions used in slope deflection equations and write the equations?

- A simply supported beam ABC is continuous over two pans AB and BC of 6m and 5m respectively. The span AB is carrying a uniformly distributed load of 20kN/m and span BC is carrying a point load of 50kN at a distance of 2m from B. Find the support moment at B. Also draw the bending moment diagram. Use slope deflection method.

- A Continuous beam is fixed at A and is supported over rollers at B and C. AB=BC=12M.The beam carries a uniformly distributed load of 30kN/m over AB and a point load of 240kN at a distance of 4M from B on span BC.B has an settlement of 30mm.E= 2 x105 N/mm2 ,I= 2 x 109 mm4 .Analyse the beam by slope deflection method.

- Analyse the beam ABCD shown in figure by Slope-Deflection method and draw bending moment diagram

- A continuous beam ABCD 12 m long is fixed at A and D, and is loaded as shown in figure. Analyze the beam completely if the following moments take place simultaneously (i) the end A yields, turning through 1/250 radians in a clock-wise direction (ii) end B sinks 30 mm in downward direction, (iii) end C sinks 20 mm in downward direction. The beam has constant I=33.20x105 mm4 and E=2x105 N/mm2. Use slope-deflection method.

- Analyse the two-span continuous beam loaded a s shown in figure, by slope-deflection method, if the moment of inertia is span AB is I and that of span BC is 3I. Sketch the B.M and SFD.

SWARNANDHRA COLLEGE OF ENGINEERING & TECHNOLOGY

(AUTONOMOUS)

DEPARTMENT OF CIVIL ENGINEERING

QUESTION BANK

SUBJECT: STRUCTIRAL ANALYSIS-I

Unit-5											Energy theorems

- Define Strain energy? Derive an expression for strain energy due to bending moment?

- Determine the vertical deflection of Joint ‘C’ for the truss shown in below Figure. Take A=500x10-6 m2, E=200 x106 kN/m2 are constant for all members.

- Derive the expression for strain energy of a straight prismatic bar of length L and cross sectional area A, if it is subjected to an axial force, F.

- State and prove Castigliano’s first theorem?

- Compute the vertical deflection of joint E by unit load method?

SWARNANDHRA COLLEGE OF ENGINEERING & TECHNOLOGY

(AUTONOMOUS)

DEPARTMENT OF CIVIL ENGINEERING

QUESTION BANK

SUBJECT: STRUCTIRAL ANALYSIS-I

Unit-6				Moving loads & Influence lines

- Distinguish between influence line diagram and bending moment diagram?

- An uniform load of 2 KN/m, 5m long crosses a girder of 20 m span from left to right. Calculate the maximum shear force and bending moment at a section 8 m from left hand support.

- What is the condition for absolute maximum bending moment due to moving UDL longer than span.

- A train of three wheel loads of magnitude 45 kN, 90 kN and 90 kN passes over a span of 40 m. The horizontal distance between the loads is 5m and 10m. Find the greatest bending moment.

- Define the term absolute maximum bending moment.

- An uniformly distributed load of 40 kN/m and of length 3m transverse across the span of simply supported length of 18 m. Compute the maximum bending moment at 4m from the left support and absolute bending moment.

- Draw the ILD for shear force and bending moment at any section of a simply supported beam?

- For the beam shown in the figure.

Draw the ILD for the following

- Reaction Vb at B.

- Reaction Va at A

- Shear force at D

- Bending moment at D

Find also maximum values of these quantities due to live load of 20kN/m.

******